Suggested Single-Point-Current-Meter (MCM) NetCDF format Jan 2026

Dimensions

station  Size:1

units: counts

long_name: station

longitude: -63.164932

latitiude: 44.249672

standard_name: platform

time  Size:15008

units: seconds since 1970-01-01T00:00:00Z

long_name: time

calendar: gregorian

time_zone: UTC

standard_name: time

Variables

These are the minimal suggested variable attributes that are required to be CF and BODC compliant.  Individuals can choose to add other attributes as needed.  Values in green are examples and may differ per data set, the remainder should be the same for all current meters.

- double datetime [station,time]

  - long_name: date_time

  - standard_name: time

  - units: seconds since 1970-01-01T00:00:00Z

  - sdn_parameter_urn:	SDN:P01::ELTMEP01

  - sdn_parameter_name:	Elapsed time (since 1970-01-01T00:00:00Z)

  - sdn_uom_urn:	SDN:P06::UTBB

  - sdn_uom_name;	Seconds

  - other units:

  - sdn_uom_urn:	SDN:P06::TISO

  - sdn_uom_name;	ISO8601

- double latitude [station]

- long_name: latitude

- standard_name: latitude

- units: degrees_north

- sdn_parameter_urn	SDN:P01::ALATZZ01

- sdn_parameter_name	Latitude north

- sdn_uom_urn	SDN:P06:DEGN

- sdn_uom_name	Degrees north

- double longitude [station]

  - long_name: longitude

  - standard_name: longitude

  - units: degrees_east

  - sdn_parameter_urn	SDN:P01::ALONZZ01

  - sdn_parameter_name	Longitude east

  - sdn_uom_urn	SDN:P06::DEGE

  - sdn_uom_name	Degrees east

- double depth[station] (depth of the instrument below surface measured)

  - long_name: instrument depth  or distance below the surface

  - standard_name: depth

  - units: meters

  - sdn_parameter_urn	SDN:P01::ADEPZZ01

  - sdn_parameter_name	Depth below surface of the water body

  - sdn_uom_urn	SDN:P06:ULAA

  - sdn_uom_name	Metres

- double pressure[time,station]  (pressure measured by instrument)

  - long_name: sea water pressure

  - standard_name: sea_water_pressure

  - units: decibars

  - _FillValue: 1e+35

  - sdn_parameter_urn	SDN:P01::PRSTPR01

  - sdn_parameter_name	Pressure (measured variable) exerted by the water body plus atmosphere by semi-fixed in-situ pressure sensor

  - sdn_uom_urn	SDN:P06:UPDB

  - sdn_uom_name	Decibars

- double temperature[time,station]

  - long_name: temperature

  - standard_name: sea_water_temperature

  - units: degree_C

  - _FillValue: 1e+35

  - sdn_parameter_urn: SDN:P01::TEMPPR01

  - sdn_parameter_name: Temperature of the water body

  - sdn_uom_urn	SDN:P06::UPAA

  - sdn_uom_name	Degrees Celsius

- double conductivity[time,station]

  - long_name: conductivity

  - standard_name: sea_water_electrical_conductivity

  - units: S m-1

  - _FillValue: 1e+35

  - sdn_parameter_urn: SDN:P01::CNDCZZ01

  - sdn_parameter_name: Electrical conductivity of the water body

  - sdn_uom_urn	SDN:P06::UECA

  - sdn_uom_name	Siemens per metre

- double HCSP[time,station]   (Contiguous storage)

- long_name: speedHorizontal

- standard_name: sea_water_speed

- units: m s-1

- _FillValue: 1e+35

- sdn_parameter_urn: SDN:P01::LCSAZZ01

- sdn_parameter_name: Current speed in the water body

- sdn_uom_urn: SDN:P06::UVAA

- sdn_uom_name: metres per second

- double HCDT[time,station]   (Contiguous storage)

- long_name: directionTrue

- standard_name: direction_of_sea_water_velocity

- units: degrees

- _FillValue: 1e+35

- sdn_parameter_urn: SDN:P01::LCDAZZ01

- sdn_parameter_name: Current direction in the water body with correction to true North

- sdn_uom_urn: SDN:P06::UAAA

- sdn_uom_name: Degrees

- double EWCT[time,station]   (Contiguous storage)

- long_name: u

- standard_name: eastward_sea_water_velocity

- units: m s-1

- _FillValue: 1e+35

- sdn_parameter_urn: SDN:P01::LCEWZZ01

- sdn_parameter_name: Eastward current velocity in the water body

- sdn_uom_urn: SDN:P06::UVAA

- sdn_uom_name: metres per second

- double NSCT[time,station]   (Contiguous storage)

- long_name: v

- standard_name: northward_sea_water_velocity

- units: m s-1

- _FillValue: 1e+35

- units: m s-1

- sdn_parameter_urn: SDN:P01::LCNSZZ01

- sdn_parameter_name: Northward current velocity in the water body

- sdn_uom_urn: SDN:P06::UVAA

- sdn_uom_name: metres per second

Other Possibilities

- double salinity[time,station]

  - long_name: salinity

  - standard_name: sea_water_practical_salinity

  - units:  psu

  - _FillValue: 1e+35

  - sdn_parameter_urn: SDN:P01::PSLTZZ01

  - sdn_parameter_name: Practical salinity of the water body

  - sdn_uom_urn: UUUU

  - sdn_uom_name: dimensionless

- double potential temperature [time,station]

  - long_name: theta

  - standard_name: sea_water_potential_temperature

  - units:  degree_C

  - _FillValue: 1e+35

  - sdn_parameter_urn: SDN:P01::POTMCV01

  - sdn_parameter_name: Potential temperature of the water body by computation using UNESCO 1983 algorithm

  - sdn_uom_urn	SDN:P06::UPAA

  - sdn_uom_name	Degrees Celsius

- double sigmatheta [time,station]

  - long_name: sigmaTheta

  - standard_name: sea_water_sigma_theta

  - units:  kg m-3

  - _FillValue: 1e+35

  - sdn_parameter_urn: SDN:P01::SIGTEQ01

  - sdn_parameter_name: Sigma-theta of the water body by computation from salinity and potential temperature using UNESCO algorithm

  - sdn_uom_urn: UKMC

  - sdn_uom_name: kilograms per cubic metresdn_parameter_urn:

- int qc_temperature[time,station]

  - long_name = "quality flag for temperature "

  - standard_name = "quality_flag"

  - flag_values = 0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 'A', 'B', 'Q' ;

  - flag_meanings = "no_quality_control, good_value,  probably_good_value, probably_bad_value, bad_value, changed_value,  value_below_detection, value_in_excess,  interpolated_value,  missing_value,  value_uncertain, nominal_value below_limit_of_quantification"

Global attributes :

Values in green are examples and may differ per data set, the remainder should be the same for all current meters.

- Conventions: CF-1.6,ACDD-1.3,IOOS-1.2

- standard_name_vocabulary: CF Standard Name Table v80

- date_created: 2025-07-29T00:00:00

- date_modified: 2025-07-29T00:00:00

- inst_type: moored Single-point Current Meter

- sampling_interval:

- chief_scientist:

- cruise_name:

- cruise_number:

- Event number

- mooring_number:

- cdm_data_type: station

- serial_number:

- data_type: moored Single-point Current Meter

- featureType: timeseries

- longitude:

- latitude:

- depth_off_bottom:

- sounding:

- instrument_depth:

- source: file name

- institution: Bedford Institute of Oceanography

- sdn_institution_id: SDN:EDMO::1811

- sdn_institution_vocabulary: https://edmo.seadatanet.org, EUROPEAN DIRECTORY OF MARINE ORGANISATIONS (EDMO)

- country_code: 1810

- sdn_country_id: SDN:C18::18

- sdn_country_vocabulary: http://vocab.nerc.ac.uk/collection/C18/current/instrument_depth

- owner_name: Fisheries and Oceans Canada (DFO)

- owner_country: Canada

- owner_email: BIO.Datashop@dfo-mpo.gc.ca

- owner_institution: Bedford Institute of Oceanography

- owner_type: institution

- owner_sector: gov_federal

- owner_country: Canada

- owner_city: Dartmouth

- owner_url: https://www.bio.gc.ca/index-en.php

- sdn_ owner _id: SDN:EDMO::1811

- originator_type: institution

- originator_name: Fisheries and Oceans Canada (DFO)

- originator_country: Canada

- originator_email: BIO.Datashop@dfo-mpo.gc.ca

- originator_address: P.O. Box 1006, 1 Challenger Dr.

- originator_city: Dartmouth

- originator_sector: gov_federal

- originator_url: https://www.bio.gc.ca/index-en.php

- sdn_originator_id: SDN:EDMO::1811

- license: Open Government Licence - Canada, https://open.canada.ca/en/open-government-licence-canada

- naming_authority: ca.gc.bio

- platform: mooring

- platform_name: mooring#

- platform_id: mooring#

- sdn_platform_id: SDN:L06::48, SDN:L06::43

- sdn_platform_vocabulary: https://vocab.nerc.ac.uk/collection/L06/current/

- deployment_platform_name: RV Endeavor (US), 32EV, http://vocab.nerc.ac.uk/collection/C17/current/32EV/

- sdn_deployment_platform_id: SDN:C17::32EV

- sdn_deployment_platform_vocabulary: https://vocab.nerc.ac.uk/collection/C17/current/

- instrument_model: RCM-8

- instrument: AANDERAA Current Meter, model number RCM-8, serial number 10471

- sdn_device_instrument_id: SDN:L22::TOOL0211

- sdn_device_instrument_vocabulary: http://vocab.nerc.ac.uk/collection/L22/current/

- sdn_device_category_id: SDN:L05::114

- sdn_device_category_vocabulary: http://vocab.nerc.ac.uk/collection/L05/current

- location_description: Scotian Shelf

- geospatial_lat_min: 44.248158

- geospatial_lat_max: 44.248158

- geospatial_lat_units: degrees_north

- geospatial_lon_min: -63.16978

- geospatial_lon_max: -63.16978

- geospatial_lon_units: degrees_east

- geospatial_vertical_units: metres

- geospatial_vertical_positive: down

- time_coverage_duration: P294DT23H30M0S

- time_coverage_start: 2017-11-24T20:02:33

- time_coverage_end: 2018-09-15T20:28:00

- title: DFO BIO,Atlantic Zonal Monitoring Program (AZMP),Nova Scotia Current Mooring Program, moored current meter

- summary: Bedford Institute of Oceanography ,Atlantic Zonal Monitoring Program (AZMP), Fall 2017,Nova Scotia Current Monitoring Program,moored ADCP model RD Instruments,from 2017-11-24 20:02:33 UTC to 2018-09-15 20:28:00 UTC

- program: Atlantic Zonal Monitoring Program (AZMP)

- project: Nova Scotia Current Monitoring Program

- keywords:  Oceans > ocean circulation > ocean currents> time-series>mooring

- processing_level: Pre and Post deployment records removed and spike removal

- flag_meaning: no_quality_control good_value probably_good_value probably_bad_value bad_value changed_value value_below_detection value_in_excess interpolated_value missing_value

- flag_references: http://www.iode.org/mg22

- flag_values: 0, 1, 2, 3, 4, 5, 6, 7, 8, 9

- processing_history:

- comment: